HSA extension

AMD AQL profile library

API Specification

rev 1.2.0

Advanced Micro Devices Corp. 2017

All rights reserved.

# Revision history

| Version | Date | Authors | Description |
| --- | --- | --- | --- |
| 1.0.0 | 5/17/2017 | Evgeny Shcherbakov | Initial version |
| 1.0.1 | 5/22/2017 | Evgeny Shcherbakov | Minor changes and code comments |
| 1.1.0 | 6/6/2017 | Evgeny Shcherbakov | Name change to “HAS extension AMD AQL profile” and minor fixes |
| 1.2.0 | 6/26/2017 | Evgeny Shcherbakov | “has_ven_amd_aqlprofile” prefix, “error_string” API |

Contents

Revision history	2

1.	High level overview	2

2.	Library implementation requirements	2

3.	Application responsibilities and requirements	3

4.	Library basic usage flow	3

5.	Library API	4

5.1.	Description	4

5.2.	Supported profiling features	4

5.3.	Profiling events	5

5.4.	Profiling parameters	5

5.5.	Profile context object	5

5.6.	AQL packets populating methods	6

5.7.	Get profile info	7

6.	Application code examples	7

6.1.	PMC profiling	7

6.2.	SQTT profiling	9

- High level overview

The library provides a C based API for helper functionality to enable a profiling of GPU compute applications. The profiling includes collecting of Performance Counters (PMC) and SQ Thread Trace (SQTT) data.

An HSA application can use the library for generating HSA_PACKET_TYPE_VENDOR_SPECIFIC AQL packets which should be submitted by the application to configure, start and stop the profiling. The library is written on C and doesn’t have direct dependencies on any infrastructure and implemented according to HSA AQL, GFX PM4 packets and the GFXIP specifications.

The generated VENDOR_SPECIFIC AQL packets carry a PM4 IB packet inside which refers to an actual PM4 commands buffer for programming GPU PMC/SQTT.

- Library implementation requirements

  - The library provides a C based API generic for PMC and SQTT profiling and supports both simultaneously.

  - The library provides an API for querying needed buffers allocations attributes. An application performs all actual memory allocations. It is expected that the application is aware of GPU PMC/SQTT capabilities and configurations.

  - The library is implemented according to HSA AQL, GFX PM4 packets and the GFXIP specifications. The library headers include HSA headers for using HSA status codes and the agent handle.

  - The library is stateless. It provides the data structures and the helper methods to the application and all data provided or processed by the library is allocated in the application context.

  - The library implementation is abstracted from the specific GFXIP. Internally there are separate instances of the implementation which are GFXIP specific and a caller is responsible to pass in the GPU handle.

  - The block name is the same for a block instances set, for example each block instance from the TCC block set, TCC0, TCC1, …, TCCN will have the same block name HSA_VEN_AMD_AQLPROFILE_BLOCK_NAME_TCC. The full name of a block instance is formed by the block name and the block index.

- Application responsibilities and requirements

- It is expected that the application is aware of GPU PMC/SQTT capabilities and configurations.

- The application performs all actual memory allocations, deallocations and the library provides API for querying needed allocations attributes.

- The application is responsible for instantiating a profile object and for submitting and waiting for completion of the library generated AQL packets.

- It is required that the command buffer was host and device accessible and executable. It is required that the output data buffer was device accessible.

- The block name is the same for a block instances set, for example each block instance from the TCC block set, TCC0, TCC1, …, TCCN will have the same block name HSA_VEN_AMD_AQLPROFILE_BLOCK_NAME_TCC. The full name of a block instance is formed by the block name and the block index.

- The application is responsible for copying data from the GPU memory if the profiling data was allocated in the GPU device memory.

- The application can enable SQTT+PMC profiling for SQ by submitting PMC start packet after SQTT start packet and the same is applied for stop packets.

- Library basic usage flow

GPU handle

Events list

Parameters list

profile object

profile object

- the profiling library API calls

- the application task

Legend:

STEP 5

STEP 4

STEP 3

STEP 2

STEP 1

Wait for the STOP AQL packet completion

Generate START AQL packet

Application AQL packets

Generate STOP AQL packet

Instantiate profile object

- Library API

## Description

The library provides helper methods for instantiation of the profile context object and for populating of the start and stop AQL packets. The profile object contains a profiling events list and needed for profiling buffers descriptors, a command buffer and an output data buffer. To check if there was an error the library methods return a status code. Also the library provides methods for querying required buffers attributes, to validate the event attributes and to get profiling output data.

Returned API status:

- hsa_status_t – HSA status codes are used from hsa.h header

Supported profiling features:

- hsa_ven_amd_aqlprofile_event_type_t – supported profiling events

- hsa_ven_amd_aqlprofile_block_name_t – supported performance counters blocks

Profiling events:

- hsa_ven_amd_aqlprofile_event_t – PMC event object structure

- hsa_ven_amd_aqlprofile_validate_event – method to check if event is valid for the specific GPU

- The counter ID values should be used from GFXIPs perfcounter users guides which is the counters select value, “Performance Counters Selection” chapter

Profiling parameters

- hsa_ven_amd_aqlprofile_parameter_name_t – supported profiling parameters

- hsa_ven_amd_aqlprofile_parameters_t - profile parameter object

Profile context object and instantiation methods:

- hsa_ven_amd_aqlprofile_descriptor_t – buffer descriptor

- hsa_ven_amd_aqlprofile_t – profile context object structure, contains a profiling events list and needed for profiling buffers descriptors, a command buffer and an output data buffer

AQL packets populating methods:

- hsa_ven_amd_aqlprofile_packet_t - vendor specific profiling AQL packet

- hsa_ven_amd_aqlprofile_start – populating the START AQL packet

- hsa_ven_amd_aqlprofile_stop – populating the STOP AQL packet

- hsa_ven_amd_aqlprofile_legacy_get_pm4 – converting of the profiling AQL packet to PM4 packet, GFX8 support

Get profile info:

- hsa_ven_amd_aqlprofile_info_type_t - supported system profile info types, like the command buffer size and the profiling PMC results

- hsa_ven_amd_aqlprofile_info_data_t – profile generic output data

- hsa_ven_amd_aqlprofile_data_callback_t - definition of output data iterator callback

- hsa_ven_amd_aqlprofile_get_info - method for getting the profile info

- hsa_ven_amd_aqlprofile_iterate_data - method for iterating the events output data

Returning the error string method:

- has_ven_amd_aqlprofile_error_string – method for returning the API error string

## Supported profiling features

Supported profiling events:

typedef enum {

HSA_VEN_AMD_AQLPROFILE_EVENT_TYPE_PMC,

HSA_VEN_AMD_AQLPROFILE_EVENT_TYPE_SQTT

} hsa_ven_amd_aqlprofile_event_type_t;

Supported performance counters blocks:

typedef enum {

HSA_VEN_AMD_AQLPROFILE_BLOCK_NAME_SQ,

HSA_VEN_AMD_AQLPROFILE_BLOCK_NAME_TCC,

…

HSA_VEN_AMD_AQLPROFILE_BLOCKS_NUMBER

} hsa_ven_amd_aqlprofile_block_name_t;

The block ID is the same for a block instances set. For example each block instance from the TCC block set, TCC0, TCC1, …, TCCN have the same block ID HSA_VEN_AMD_AQLPROFILE_BLOCK_NAME_TCC.

## Profiling events

PMC event object structure:

typedef struct {

hsa_ven_amd_aqlprofile_block_name_t block_name;

uint32_t block_index;

uint32_t counter_id,

} hsa_ven_amd_aqlprofile_event_t;

‘counter_id’ value is specified in GFXIPs perfcounter user guides which is the counters select value, “Performance Counters Selection” chapter.

Check if event is valid for the specific GPU:

hsa_status_t hsa_ven_amd_aqlprofile_validate_event(

hsa_agent_t agent,				// HSA handle for the profiling GFXIP

const hsa_ven_amd_aqlprofile_event_t * event,	// Pointer on a validated event

bool * result);					// True if the event is valid, False otherwise

## Profiling parameters

All parameters are generic and if not applicable for a specific profile configuration then error status will be returned.

Supported profiling parameters:

typedef enum {

// SQTT applicable parameters

HSA_VEN_AMD_AQLPROFILE_PARAMETER_NAME_TOKEN_MASK,

HSA_VEN_AMD_AQLPROFILE_PARAMETER_NAME_REG_MASK,

HSA_VEN_AMD_AQLPROFILE_PARAMETER_NAME_VM_ID_MASK,

HSA_VEN_AMD_AQLPROFILE_PARAMETER_NAME_INSTRUCTION_MASK,

HSA_VEN_AMD_AQLPROFILE_PARAMETER_NAME_COMPUTE_UNIT_TARGET,

HSA_VEN_AMD_AQLPROFILE_PARAMETER_NAME_SHADER_ARRAY_TARGET,

HSA_VEN_AMD_AQLPROFILE_PARAMETER_NAME_SIMD_MASK,

HSA_VEN_AMD_AQLPROFILE_PARAMETER_NAME_USER_DATA,

HSA_VEN_AMD_AQLPROFILE_PARAMETER_NAME_CAPTURE_MODE,

HSA_VEN_AMD_AQLPROFILE_PARAMETER_NAME_IS_WRAPPED,

HSA_VEN_AMD_AQLPROFILE_PARAMETER_NAME_RANDOM_SEED	// Seems to be obsolete

// and a candidate for removal

} hsa_ven_amd_aqlprofile_parameter_name_t;

Profile parameter object:

typedef struct {

hsa_ven_amd_aqlprofile_parameter_name_t parameter_name;

uint32_t value;

} hsa_ven_amd_aqlprofile_parameters_t;

## Profile context object

The library provides a profile object structure which contains the events array, a buffer for the profiling start/stop commands and a buffer for the output data. The buffers are specified by the buffer descriptors and allocated by the application.

The buffers allocation attributes, the command buffer size, the PMC output buffer size as well as profiling output data can be get using the generic get profile info helper method hsa_ven_amd_aqlprofile_get_info.

Buffer descriptor:

typedef struct {

void * ptr;

uint32_t size;

} hsa_ven_amd_aqlprofile_descriptor_t;

Profile context object structure, contains profiling events list and needed for profiling buffers descriptors, a command buffer and an output data buffer:

typedef struct {

hsa_agent_t agent;					// GFXIP handle

hsa_ven_amd_aqlprofile_event_type_t type;		// Events type

const hsa_ven_amd_aqlprofile_event_t * events;		// Events array

uint32_t event_count;					// Events count

const hsa_ven_amd_aqlprofile_parameters_t * parameters;	// Parameters array

uint32_t parameter_count;				// Parameters count

hsa_ven_amd_aqlprofile_descriptor_t output_buffer;	// Output data buffer

hsa_ven_amd_aqlprofile_descriptor_t command_buffer;	// PM4 commands buffer

} hsa_ven_amd_aqlprofile_t;

## AQL packets populating methods

The helper methods to populate provided by the application START and STOP AQL packets which the application is required to submit before and after profiled GPU task packets respectively.

Vendor specific profiling AQL packet:

typedef struct {

uint16_t header;

…

hsa_signal_t completion_signal;

} hsa_ven_amd_aqlprofile_packet_t;

The format field in the profile AQL packets header set to VENDOR_SPECIFIC, which is zero.

Method to populate the provided AQL packet with profiling start commands:

hsa_status_t hsa_ven_amd_aqlprofile_start(

const hsa_ven_amd_aqlprofile_t * profile,			// [in] profile context object

hsa_ven_amd_aqlprofile_packet_t * aql_start_packet);	// [out] profile start AQL packet

Method to populate the provided AQL packet with profiling stop commands:

hsa_status_t hsa_ven_amd_aqlprofile_stop(

const hsa_ven_amd_aqlprofile_t * profile, 		// [in] profile context object

hsa_ven_amd_aqlprofile_packet_t * aql_stop_packet);	// [out] profile stop AQL packet

Converting of the profiling AQL packet to PM4 packet, GFX8 support:

#define HSA_VEN_AMD_AQLPROFILE_LEGACY_PM4_PACKET_SIZE 64

hsa_status_t hsa_ven_amd_aqlprofile_legacy_get_pm4(

const hsa_ven_amd_aqlprofile_packet_t * aql_packet,	// [in] AQL profile start/stop packet

void * pm4);						// [out] profile legacy PM4 packet blob

## Get profile info

Generic method for getting various profile info including profile buffers attributes like the command buffer size and the profiling results.

It’s implied that all counters are 64bit values.

Profile attributes:

typedef union {

HSA_VEN_AMD_AQLPROFILE_INFO_TYPE_COMMAND_BUFFER_SIZE,	// get_info returns uint32_t value

HSA_VEN_AMD_AQLPROFILE_INFO_TYPE_PMC_BUFFER_SIZE,		// get_info returns uint32_t value

HSA_VEN_AMD_AQLPROFILE_INFO_TYPE_PMC_DATA,			// get_info returns PMC uint64_t value

// in info_data_t object

HSA_VEN_AMD_AQLPROFILE_INFO_TYPE_SQTT_DATA			// get_info returns SQTT buffer ptr/size

// in info_data_t object

} hsa_ven_amd_aqlprofile_info_type_t;

Profile generic output data:

typedef struct {

uint32_t sample_id;					// PMC sample or SQTT buffer index

union {

struct {

hsa_ven_amd_aqlprofile_event_t event;		// PMC event

uint64_t result;					// PMC result

} pmc_data;

hsa_ven_amd_aqlprofile_descriptor_t sqtt_data;		// SQTT output data descriptor

};

} hsa_ven_amd_aqlprofile_info_data_t;

Definition of output data iterator callback:

hsa_status_t (*hsa_ven_amd_aqlprofile_data_callback_t)(

hsa_ven_amd_aqlprofile_info_type_t info_type,		// [in] data type. PMC or SQTT data

hsa_ven_amd_aqlprofile_info_data_t * info_data,		// [in] info data object

void * callback_data);					// [in/out] data pointer passed to the callback

Method for getting the profile info:

hsa_status_t hsa_ven_amd_aqlprofile_get_info(

const hsa_ven_amd_aqlprofile_t * profile,			// [in] profile context object

hsa_ven_amd_aqlprofile_info_type_t attribute,		// [in] requested profile attribute

void * value);						// [in/out] returned value

Method for iterating the events output data:

hsa_status_t hsa_ven_amd_aqlprofile_iterate_data(

const hsa_ven_amd_aqlprofile_t * profile,			// [in] profile context object

hsa_ven_amd_aqlprofile_data_callback_t callback;		// [in] callback to iterate the output data

void * callback_data);					// [in/out] data passed to the callback

## Returning the error string method

hsa_status_t has_ven_amd_aqlprofile_error_string(

const char ** str);					// [out] the API error string pointer returning

- Application code examples

## PMC profiling

hsa_status_t status;

hsa_agent_t agent;

hsa_ven_amd_aqlprofile_t profile;

hsa_ven_amd_aqlprofile_event_t event;

hsa_ven_amd_aqlprofile_packet_t * start_packet;

hsa_ven_amd_aqlprofile_packet_t * stop_packet;

hsa_ven_amd_aqlprofile_info_data_t sample;

uint32_t command_buffer_alignment;

uint32_t command_buffer_size;

uint32_t output_buffer_alignment;

uint32_t output_buffer_size;

// Resulted PMC sample value

uint64_t result;

// GPU identificator

hsa_iterate_agents(<get agent callback>, &agent);

// Instantiation of the profile object //////////////////////////////////////////////////////////////

// Set the event fields

event.block_name = HSA_VEN_AMD_AQLPROFILE_BLOCK_NAME_SQ;

event.block_index = 0;

event.counter_id = <some counter ID>;

// Initialization the profile

memset(&profile, 0, sizeof(profile));

profile.agent = agent;

profile.type = HSA_VEN_AMD_AQLPROFILE_EVENT_TYPE_PMC;

// set enabled events list, one event in this example

profile.events = &event;

profile.event_count = 1;

// Profile buffers attributes

command_buffer_alignment = <command buffer alignment>;

status = hsa_ven_amd_aqlprofile_get_info(

&profile,

HSA_VEN_AMD_AQLPROFILE_INFO_TYPE_COMMAND_BUFFER_SIZE,

&command_buffer_size);

<check the status>

output_buffer_alignment = <PMC output data alignment>;

status = hsa_ven_amd_aqlprofile_get_info(

&profile,

HSA_VEN_AMD_AQLPROFILE_INFO_TYPE_PMC_DATA_SIZE,

&output_buffer_size);

<check the status>

// Application is allocating the command buffer and output buffer

profile.command_buffer.ptr = allocate(

command_buffer_alignment,

command_buffer_size,

MODE_HOST_ACC|MODE_DEV_ACC|MODE_EXEC_DATA);

profile.command_buffer.size = command_buffer_size;

profile.output_buffer.ptr = allocate(

output_buffer_alignment,

output_buffer_size,

MODE_HOST_ACC|MODE_DEV_ACC);

profile.output_buffer.size = output_buffer_size;

// Generation of the START AQL packet //////////////////////////////////////////////////////////////

<enqueuing AQL start_paket>

// Populating the AQL start packet

status = hsa_ven_amd_aqlprofile_start(&profile, start_packet);

<check the status>

<enqueuing AQL packets of the application task>

// Generation of the STOP AQL packet     //////////////////////////////////////////////////////////////

<enqueuing AQL stop_paket>

// Populating the AQL stop packet

status = hsa_ven_amd_aqlprofile_stop(&profile, stop_packet);

<check the status>

<wait for the AQL stop_paket completion>

// Query PMC sample for shader 0 as sample.pmc_data.result

sample.sample_id = 0;

sample.pmc_data.event = event;

hsa_ven_amd_aqlprofile_get_info(

&profile,

HSA_VEN_AMD_AQLPROFILE_INFO_TYPE_PMC_DATA,

&sample);

// Final PMC sample result

result = sample.pmc_data.result;

## SQTT profiling

hsa_status_t status;

hsa_agent_t agent;

hsa_ven_amd_aqlprofile_t profile;

hsa_ven_amd_aqlprofile_parameter_t parameter;

hsa_ven_amd_aqlprofile_packet_t * start_packet;

hsa_ven_amd_aqlprofile_packet_t * stop_packet;

hsa_ven_amd_aqlprofile_info_data_t sample;

uint32_t command_buffer_alignment;

uint32_t command_buffer_size;

uint32_t output_buffer_alignment;

uint32_t output_buffer_size;

// Resulted SQTT trace

hsa_ven_amd_aqlprofile_descriptor_t trace;

// GPU identificator

hsa_iterate_agents(<get agent callback>, &agent);

// Instantiation of the profile object //////////////////////////////////////////////////////////////

// Set the parameter fields

parameter.name = HSA_VEN_AMD_AQLPROFILE_PARAMETER_NAME_INSTRUCTION_MASK;

parameter.value = <some trace instructions mask>;

// Initialization the profile

memset(&profile, 0, sizeof(profile));

profile.agent = agent;

profile.type = HSA_VEN_AMD_AQLPROFILE_EVENT_TYPE_SQTT;

// set parameters list, one parameter in this example

profile.parameters = &parameter;

profile.parameter_count = 1;

// Profile buffers attributes

command_buffer_alignment = <command buffer alignment>;

status = hsa_ven_amd_aqlprofile_get_info(

&profile,

HSA_VEN_AMD_AQLPROFILE_INFO_TYPE_COMMAND_BUFFER_SIZE,

&command_buffer_size);

<check the status>

output_buffer_alignment = <SQTT output data alignment>;

output_buffer_size = <SQTT data application estimated size>;

// Application is allocating the command buffer and output buffer

profile.command_buffer.ptr = allocate(

command_buffer_alignment,

command_buffer_size,

MODE_EXEC_DATA|MODE_HOST_ACC|MODE_DEV_ACC);

profile.command_buffer.size = command_buffer_size;

profile.output_buffer.ptr = allocate(

output_buffer_alignment,

output_buffer_size,

MODE_DEV_MEM);

profile.output_buffer.size = output_buffer_size;

// Generation of the START AQL packet //////////////////////////////////////////////////////////////

<enqueuing AQL start_packet>

// Populating the AQL start packet

status = hsa_ven_amd_aqlprofile_start(&profile, start_packet);

<check the status>

<enqueuing AQL packets of the application task>

// Generation of the STOP AQL packet     //////////////////////////////////////////////////////////////

<enqueuing AQL stop_paket>

// Populating the AQL stop packet

status = hsa_ven_amd_aqlprofile_stop(&profile, stop_packet);

<check the status>

<wait for the AQL stop_packet completion>

// Query SQTT trace for shader engine 0 as sample.sqtt_data

sample.sample_id = 0;

hsa_ven_amd_aqlprofile_get_info(

&profile,

HSA_VEN_AMD_AQLPROFILE_INFO_TYPE_PMC_DATA,

&sample);

// Final SQTT trace descriptor

trace = sample.sqtt_data;